# Residential Remodel Submittal Checklist and Standards

## Submittal Checklist:

### Drawings:

- Site Plan – Must show the location of the structure on the site and any existing structures, dimensional setbacks from the back of sidewalk to the new structure or a note stating setbacks are measured from the back of sidewalk, dimensional width of driveway and any proposed or existing fence.

- Floor Plan – All rooms and areas must be easily identified with labels and clear dimensions, and ceiling heights. Provide Floor plan identifying what was existing and what is new.

- Roof and Floor Framing (if applicable) – Must specify the type, size, spacing and spans and provide compliant roof ventilation.

- Wall Bracing – Must include the method type, fastening information, portal frame type with detail, clearly identified braced wall lines and lengths of each and all corner returns.

- Structural Drawings (if applicable) – Must be stamped with current stamp, signed and dated by an Idaho licensed professional.

### Documents:

- Structural Calculations (if applicable) – Must be stamped with current stamp, signed and dated by an Idaho licensed professional.

## Submittal Standards and File Naming Requirements:

### All drawing sheets must be submitted:

- In a PDF format

- In landscape format in the horizontal position

- With a space reserved in the upper left corner for City Approval stamps

- Drawn to scale with each sheet stating the scale

- As individual sheets

- With a file name allowing for easy identification of the drawing content

  - Example: CS – Cover Sheet; SP1 – Site Plan; S4.0 – Structural Roof Framing; E2.0 – Electrical Schedule

- As print ready, i.e., set up properly for printing with title block, no data outside the print page area, etc.

### All documents (non-drawing files) must be submitted:

- As searchable PDF files

- With the stamp of an Idaho Licensed Engineer for structural calculations and geotechnical investigation reports