# Residential Addition Submittal Checklist and Standards

## Submittal Checklist:

### Drawings:

- Site Plan – Must show the location of the structure on the site and any existing structures, dimensional setbacks from the back of sidewalk to the new structure or a note stating setbacks are measured from the back of sidewalk, dimensional width of driveway and any proposed or existing fence.

- Foundation Plan – Must include the type of foundation system, compliant crawlspace venting, anchor bolt locations and a footing detail.

  - Projects including basements require submittal of a soils report - Soils and Geotech report for support of the structure must be stamped with current stamp and signed by an Idaho licensed engineer.

- Floor Plan – All rooms and areas must be easily identified with labels and clear dimensions, and ceiling heights.

- Roof and Floor Framing – Must specify the type, size, spacing and spans and provide compliant roof ventilation.

- Elevations – must include front, back and right, left sides.

- Wall Bracing – Must include the method type, fastening information, portal frame type with detail, clearly identified braced wall lines and lengths of each and all corner returns.

- Wall Sections and Details – An elevation wall detail is required.

  - An elevation wall detail example is shown on the Typical 1 Story Wall Section Detail.

- Connection Details – Required for all exterior wall attachments such as a deck, patio cover, lean-to, etc.

- Structural Drawings (if applicable) – Must be stamped with current stamp, signed and dated by an Idaho licensed professional.

### Documents:

- Energy Compliance – Alternate Energy Path (must be signed and include all pages) or Prescriptive Method.

- Structural Calculations (if applicable) – Must be stamped with current stamp, signed and dated by an Idaho licensed professional.

- Tall Wall Engineering (if applicable) – Must be stamped with current stamp, signed and dated by an Idaho licensed professional.

## Submittal Standards and File Naming Requirements:

### All drawing sheets must be submitted:

- In a PDF format

- In landscape format in the horizontal position

- With a space reserved in the upper left corner for City Approval stamps

- Drawn to scale with each sheet stating the scale

- As individual sheets

- With a file name allowing for easy identification of the drawing content.

Example: CS – Cover Sheet; SP1 – Site Plan; S4.0 – Structural Roof Framing; E2.0 – Electrical Schedule

- As print ready, i.e., set up properly for printing with title block, no data outside the print page area, etc.

### All documents (non-drawing files) must be submitted:

- As searchable PDF files

- With the stamp of an Idaho Licensed Engineer for structural calculations and geotechnical investigation reports